PUP MANET PROJECT
MANET ROUTING PROTOCOL DOCUMENTATION

Current Routing Architecture and Message Routing Operation

Generated Date: May 11, 2026

# 1. Overview

The PUP MANET project uses a hybrid MANET routing architecture designed for emergency and off-grid
communication using ESP32 LoRa nodes, Raspberry Pi gateways, Android mobile devices, and WiFi-based
network bridging.

The current implementation does not use a full dynamic MANET routing protocol such as AODV or OLSR.
Instead, the system uses a controlled reliable mesh-routing approach optimized for:
- LoRa communication
- ESP32 hardware limitations
- Reliable message delivery
- Gateway-assisted routing
- Delay-tolerant communication
- Simplified debugging and testing

The routing system focuses on stable and predictable message forwarding between nodes while maintaining
low routing overhead suitable for LoRa communication.

# 2. Routing Protocol Used

The current routing implementation uses:

Controlled Reliable Mesh Routing with Gateway-Assisted Store-and-Forward Logic

Protocol Classification:
- Hybrid MANET Routing
- Delay-Tolerant Mesh Routing
- Multi-Hop Store-and-Forward Routing

Communication Layers:
- Bluetooth communication between Android devices and ESP32 nodes
- LoRa communication between ESP32 MANET nodes
- WiFi bridge communication between Raspberry Pi gateways

# 3. Network Architecture

The MANET routing system is composed of the following layers:

A. Android Messaging Layer
- Android phones connect to nearby ESP32 nodes using Bluetooth.
- Users can send and receive messages through the Android application.

B. ESP32 LoRa MANET Layer
- ESP32 nodes communicate using LoRa radio modules.
- Nodes perform direct peer-to-peer communication.
- Messages are forwarded hop-by-hop between nodes.

C. Raspberry Pi Gateway Layer
- Raspberry Pi gateways connect separate MANET clusters.
- Gateways perform message buffering and forwarding.
- Gateways transfer messages between isolated MANET networks.

D. WiFi Inter-Network Bridge Layer
- WiFi routers connect separate MANET groups.
- This layer enables inter-network communication between remote MANET clusters.

# 4. Routing Operation

The routing system uses deterministic message forwarding.

Step 1 – Neighbor Discovery
- ESP32 nodes periodically transmit HELLO packets.
- Neighbor nodes monitor RSSI and signal quality.

Step 2 – Route Monitoring
- Nodes continuously monitor:
  • RSSI
  • Packet acknowledgement success
  • Link stability

Step 3 – Local Message Routing
- Messages destined for nodes inside the same MANET cluster are forwarded through LoRa hops.

Step 4 – Gateway Routing
- Messages intended for another MANET cluster are forwarded to the Raspberry Pi gateway.
- The gateway transfers the message through the WiFi bridge to another gateway.

Step 5 – Remote MANET Delivery
- The destination gateway forwards the message into the remote MANET cluster through LoRa.

Step 6 – Final Delivery
- The destination ESP32 node forwards the message to the Android device through Bluetooth.

# 5. Store-and-Forward Operation

The routing system uses store-and-forward message handling.

If a route temporarily fails:
- Messages are stored locally.
- Retransmission attempts continue automatically.
- Messages remain queued until acknowledgement is received.

This method improves reliability in:
- Weak signal environments
- Intermittent LoRa connectivity
- Temporary network degradation
- Mobile node movement scenarios

# 6. Packet Types

| Packet Type | Purpose | Stored | Function |
| --- | --- | --- | --- |
| HELLO | Neighbor discovery | Latest only | Detect nearby nodes and monitor link quality |
| MSG | User message transport | Yes | Transfers text messages across the MANET |
| ACK | Delivery acknowledgement | Yes | Confirms successful message delivery |
| ROUTE_ADV | Gateway advertisement | Latest only | Advertises reachable MANET networks |
| NACK | Delivery failure notification | Yes | Triggers retransmission handling |

# 7. Route Monitoring and Failover

The routing system continuously monitors route quality.

Metrics monitored include:
- RSSI
- Signal stability
- Packet acknowledgement success rate

If a route degrades:
- Messages are redirected through alternate available paths.
- Gateways continue buffering and forwarding messages.
- Once the route stabilizes, normal routing resumes automatically.

This mechanism allows the MANET system to continue operating during:
- Weak LoRa signals
- Partial network outages
- Node movement
- Temporary communication interruptions

# 8. End-to-End Message Flow

Android Device A
    ↓ Bluetooth
ESP32 Node A1
    ↓ LoRa
ESP32 Node A2
    ↓ LoRa
Raspberry Pi Gateway A
    ↓ WiFi Bridge
Raspberry Pi Gateway B
    ↓ LoRa
ESP32 Node B2
    ↓ Bluetooth
Android Device B

This routing structure enables communication between separate MANET clusters even without
internet connectivity or cellular coverage.

# 9. Conclusion

The PUP MANET project currently implements a controlled reliable mesh-routing architecture using:
- ESP32 LoRa nodes
- Raspberry Pi gateways
- WiFi inter-network bridging
- Store-and-forward message delivery

The routing system provides:
- Multi-hop LoRa communication
- Neighbor discovery
- Route monitoring
- Reliable message forwarding
- Gateway-assisted inter-network routing
- Delay-tolerant operation
- Automatic retransmission handling

The implementation is optimized for stable LoRa-based MANET operation while maintaining low routing
overhead suitable for ESP32 and LoRa hardware limitations.