# PUP MANET — Test Procedures

Version 1.0-working-base | June 19, 2026

# 1. Test Environment

| Item | Quantity | Purpose |
| --- | --- | --- |
| ESP32 + SX1278 | 6 | MANET nodes |
| ESP32 + SX1278 | 2 | Gateway nodes |
| Raspberry Pi 3B | 2 | Gateway services |
| Android phones (12+) | 2 | User devices |
| 18650 batteries | 6 | Node power |

# 2. Unit Tests (240/240 PASS)

Run: cd rpi-gateway && python3 -m unittest discover -s tests -p "test_*.py". All 240 tests pass in ~2.3 seconds.

| Module | Tests | Focus |
| --- | --- | --- |
| Store-Forward Queue | 35 | Enqueue, dedup, replay |
| Replay Engine | 28 | Scheduling, retry, backoff |
| Route State Machine | 46 | Failover states |
| Gateway Integration | 15 | VPN relay, ACK tracking |
| Reconnection | 7 | Reconnect hardening |

# 3. Failover Test Campaign (26/26 PASS)

Tests covering: normal operation, gateway disable, VPN disconnect, store-and-forward replay, degradation/recovery state machine. All 26 scenarios pass.

# 4. Physical Test Campaigns

| Distance | RSSI | SNR | Delivery |
| --- | --- | --- | --- |
| 1m | -38 dBm | 10.5 dB | PASS |
| 10m | -66 dBm | 8.5 dB | PASS |
| 50m | -92 dBm | 3.0 dB | 2/3 PASS |

# 5. Cross-Mesh Routing Validation

Test: Phone A (nodeA3) sends message to Phone B (nodeB2) via Gateway A → VPN → Gateway B. Both directions confirmed working. Broadcast messages propagate to all 8 nodes.

# 6. ACK Validation

ACK types: FORWARDED_OVER_LORA (ESP32 forwarded via LoRa), DELIVERED (end node confirmed receipt). 25-second timeout. >90% delivery confirmation rate on both phones.

# 7. Regression Checklist

[ ] 240 unit tests pass
[ ] 26/26 failover tests pass
[ ] 8-node discovery verified
[ ] Cross-mesh routing A-B both directions
[ ] Broadcast delivery verified
[ ] ACK: Delivered >90%
[ ] Bluetooth paired device filter working
[ ] Chat tab chips show only BT-paired nodes
[ ] All simulation UI elements hidden