# PUP MANET — Deployment Guide

Version 1.0-working-base | June 19, 2026

# 1. Prerequisites

| Item | Purpose |
| --- | --- |
| ESP32 Dev Board + SX1278 | Mesh node / gateway |
| Raspberry Pi 3B + SSD | Gateway server |
| Android phone (12+) | User device |
| PlatformIO | Firmware build/flash |
| Tailscale | Gateway VPN |

# 2. Flash ESP32 Firmware

pio run -e nodeA1_lora --target upload
pio run -e nodeA2_lora --target upload
pio run -e nodeA3_lora --target upload
pio run -e nodeB1_lora --target upload
pio run -e nodeB2_lora --target upload
pio run -e nodeB3_lora --target upload
pio run -e gatewayA_lora --target upload
pio run -e gatewayB_lora --target upload

Physically label each ESP32 with its node ID (A1-A3, B1-B3, GWA, GWB).

# 3. Gateway Setup

On each RPi:
1. sudo apt update && sudo apt upgrade
2. curl -fsSL https://tailscale.com/install.sh | sh && sudo tailscale up
3. Disable swap: sudo dphys-swapfile swapoff && sudo dphys-swapfile uninstall
4. Copy gateway_service.py and config.json to ~/raspberry-pi-gateway/
5. Start: screen -dmS gateway bash -c "cd ~/raspberry-pi-gateway && python3 gateway_service.py 2>&1 | tee gateway.log"

# 4. Install APK

cd android-chat-app
./gradlew clean :app:assembleDebug
adb install -r app/build/outputs/apk/debug/app-debug.apk

# 5. Pair ESP32 Nodes

Settings → Bluetooth → Pair new device → PUP-MANET-nodeXX → PIN: 1234. Repeat for each phone-node pairing.

# 6. System Verification

1. Power both gateways, wait 30s for VPN
2. Power all 6 nodes, wait 30s for HELLO
3. Open app → Bluetooth tab → Connect ESP32
4. Nodes tab → Refresh → verify 8 nodes
5. Chat tab → select destination → send test message
6. Verify Bridge ACK: Delivered on both phones

# 7. Power Loss Recovery

| Component | Recovery Time | Mechanism |
| --- | --- | --- |
| Gateway RPi | ~30s | SSD + ext4 journal + fsck.repair=yes |
| Node ESP32 | ~10s | Microcontroller boot |
| Node Discovery | ~30s | HELLO broadcasts every 10s |
| Tailscale VPN | ~30s | Auto-reconnect |

After power restoration, wait 60 seconds for full recovery. Refresh Nodes on both phones and verify 8-node discovery.